Produced by Ricardo-AEA on behalf of South Holland District Council

SOUTH HOLLAND WESTMERE SCHOOL

01 to 28 February 2013

These data are provisional from 01/07/2012 and may be subject to further quality control

| POLLUTANT | NOX | NO | NO2 | O3 | PM10*+ |
| --- | --- | --- | --- | --- | --- |
| Number Very High | - | - | 0 | 0 | 0 |
| Number High | - | - | 0 | 0 | 0 |
| Number Moderate | - | - | 0 | 0 | 0 |
| Number Low | - | - | 593 | 604 | 95 |
| Maximum 15-minute mean | 105 µg m-3 | 34 µg m-3 | 53 µg m-3 | 182 µg m-3 | 42 µg m-3 |
| Maximum hourly mean | 92 µg m-3 | 28 µg m-3 | 50 µg m-3 | 86 µg m-3 | 34 µg m-3 |
| Maximum running 8-hour mean | 56 µg m-3 | 20 µg m-3 | 35 µg m-3 | 84 µg m-3 | 29 µg m-3 |
| Maximum running 24-hour mean | 39 µg m-3 | 10 µg m-3 | 27 µg m-3 | 81 µg m-3 | 19 µg m-3 |
| Maximum daily mean | 37 µg m-3 | 10 µg m-3 | 26 µg m-3 | 81 µg m-3 | 15 µg m-3 |
| Average | 20 µg m-3 | 3 µg m-3 | 15 µg m-3 | 50 µg m-3 | 13 µg m-3 |
| Data capture | 88.2 % | 88.2 % | 88.2 % | 89.1 % | 15.5 % |

* PM10 Indicative Gravimetric Equivalent µgm-3

+ PM10 as measured by a TEOM using a factor of 1.3 for Indicative Gravimetric Equivalence

Particulate matter concentrations are reported at ambient temperature and pressure.

All gaseous pollutant mass units are at 20'C and 1013mb.

NOX mass units are NOX as NO2 µgm-3

| Pollutant | Air Quality (England) Regulations 2000 and (Amendment) Regulations 2002 | Exceedences | Days |
| --- | --- | --- | --- |
| Nitrogen Dioxide | Annual mean > 40 µg m-3 | - | - |
| Nitrogen Dioxide | Hourly mean > 200 µg m-3 | 0 | 0 |
| Ozone | Running 8-hour mean > 100 µg m-3 | 0 | 0 |
| PM10 Particulate Matter (Gravimetric) | Daily mean > 50 µg m-3 | 0 | 0 |
| PM10 Particulate Matter (Gravimetric) | Annual mean > 40 µg m-3 | - | - |

Note: For a strict comparison against the objectives there must be a data capture of >90% throughout the calendar year

Produced by Ricardo-AEA on behalf of South Holland District Council

South Holland Westmere School

Hourly Mean Data for 01 to 28 February 2013

Date Created: 05/03/2013

| Ian Roberts Environmental Quality Ricardo-AEA The Gemini Building Fermi Avenue Harwell Didcot Oxfordshire OX11 0QJ | Direct line 0870 190 6509 e-mail ian.roberts@ricardo-aea.com |
| --- | --- |