Produced by Ricardo-AEA on behalf of South Holland District Council

SOUTH HOLLAND WESTMERE SCHOOL

01 to 31 July 2015

These data are provisional from 01/04/2015 and may be subject to further quality control

| POLLUTANT | PM10*+ | NO2 | NOX | O3 |
| --- | --- | --- | --- | --- |
| Maximum 15-minute mean | 62 µg m-3 | 48 µg m-3 | 88 µg m-3 | 194 µg m-3 |
| Maximum hourly mean | 55 µg m-3 | 46 µg m-3 | 50 µg m-3 | 188 µg m-3 |
| Maximum running 8-hour mean | 46 µg m-3 | 37 µg m-3 | 40 µg m-3 | 175 µg m-3 |
| Maximum running 24-hour mean | 37 µg m-3 | 32 µg m-3 | 34 µg m-3 | 130 µg m-3 |
| Maximum daily mean | 36 µg m-3 | 32 µg m-3 | 34 µg m-3 | 130 µg m-3 |
| Average | 15 µg m-3 | 6 µg m-3 | 8 µg m-3 | 54 µg m-3 |
| Data capture | 99.7 % | 99.1 % | 99.1 % | 99.5 % |

* PM10 Indicative Gravimetric Equivalent µg m-3

+ PM10 as measured by a TEOM using a gravimetric factor of 1.3 for Indicative Gravimetric Equivalent

All gaseous pollutant mass units are at 20'C and 1013mb. Particulate matter concentrations are reported at ambient temperature and pressure.

NOX mass units are NOX as NO2 µg m-3

| Pollutant | Air Quality (England) Regulations 2000 and (Amendment) Regulations 2002 | Exceedences | Days |
| --- | --- | --- | --- |
| PM10 Particulate Matter (Gravimetric) | Daily mean > 50 µg m-3 | 0 | 0 |
| PM10 Particulate Matter (Gravimetric) | Annual mean > 40 µg m-3 | - | - |
| Nitrogen Dioxide | Annual mean > 40 µg m-3 | - | - |
| Nitrogen Dioxide | Hourly mean > 200 µg m-3 | 0 | 0 |
| Nitrogen Oxides (NO2) | Annual mean > 30 µg m-3 | - | - |
| Ozone | Running 8-hour mean > 100 µg m-3 | 31 | 4 |

Note: For a strict comparison against the objectives there must be a data capture of >90% throughout the calendar year

Produced by Ricardo-AEA on behalf of South Holland District Council

South Holland Westmere School

Hourly Mean Data for 01 to 31 July 2015

Date Created: 03/08/2015

| Michael Davies Environmental Quality Ricardo-AEA The Gemini Building Fermi Avenue Harwell Didcot Oxfordshire OX11 0QJ | Direct line 01235 753234 e-mail michael.a.davies@ricardo.com |
| --- | --- |