Produced by Ricardo on behalf of South Holland District Council

SOUTH HOLLAND WESTMERE SCHOOL

01 to 30 June 2016

These data are provisional from 01/04/2016 and may be subject to further quality control

| POLLUTANT | PM10*+ | NO | NO2 | NOX | O3 |
| --- | --- | --- | --- | --- | --- |
| Number Very High | 0 | - | 0 | - | 0 |
| Number High | 0 | - | 0 | - | 0 |
| Number Moderate | 0 | - | 0 | - | 0 |
| Number Low | 720 | - | 718 | - | 479 |
| Maximum 15-minute mean | 65 µg m-3 | 11 µg m-3 | 32 µg m-3 | 38 µg m-3 | 102 µg m-3 |
| Maximum hourly mean | 57 µg m-3 | 9 µg m-3 | 29 µg m-3 | 36 µg m-3 | 98 µg m-3 |
| Maximum running 8-hour mean | 30 µg m-3 | 6 µg m-3 | 24 µg m-3 | 28 µg m-3 | 92 µg m-3 |
| Maximum running 24-hour mean | 27 µg m-3 | 4 µg m-3 | 17 µg m-3 | 22 µg m-3 | 85 µg m-3 |
| Maximum daily mean | 25 µg m-3 | 3 µg m-3 | 16 µg m-3 | 18 µg m-3 | 84 µg m-3 |
| Average | 15 µg m-3 | 1 µg m-3 | 8 µg m-3 | 9 µg m-3 | 52 µg m-3 |
| Data capture | 100.0 % | 99.7 % | 99.7 % | 99.7 % | 66.1 % |

* PM10 Indicative Gravimetric Equivalent µg m-3

+ PM10 as measured by a TEOM using a gravimetric factor of 1.3 for Indicative Gravimetric Equivalent

All gaseous pollutant mass units are at 20'C and 1013mb. Particulate matter concentrations are reported at ambient temperature and pressure.

NOX mass units are NOX as NO2 µg m-3

| Pollutant | Air Quality (England) Regulations 2000 and (Amendment) Regulations 2002 | Exceedences | Days |
| --- | --- | --- | --- |
| PM10 Particulate Matter (Gravimetric) | Daily mean > 50 µg m-3 | 0 | 0 |
| PM10 Particulate Matter (Gravimetric) | Annual mean > 40 µg m-3 | - | - |
| Nitrogen Dioxide | Annual mean > 40 µg m-3 | - | - |
| Nitrogen Dioxide | Hourly mean > 200 µg m-3 | 0 | 0 |
| Ozone | Running 8-hour mean > 100 µg m-3 | 0 | 0 |

Note: For a strict comparison against the objectives there must be a data capture of >90% throughout the calendar year

Produced by Ricardo on behalf of South Holland District Council

South Holland Westmere School

Hourly Mean Data for 01 to 30 June 2016